# Production Lines in Food Industry

## Pavel Hoffman; 10th semester (summer)

#### Practice requirements for process engineers

- Logical thinking

- Creative work and problem solution

- Knowledge of solved and related problems

- Exploitation of own knowledge and the others one

- Proceed from common to detailed (black boxes, balance of an all system or line and than subsystems and apparatuses)

- In process of a trouble shooting or optimisation of a line it is necessary to mind or take into account details and trifles.

- It is important to interest in a user of your designs and proposals

- Do not hold up on one solution or proposal, but revise it according of new pieces of knowledge or situation development.

- To have in reserve several alternative solutions or variants for a case that there are problems with an original ones.

- A bad operation (troubles) of a apparatus, line etc. usually has more causes, not only one.

- It is necessary to check given data, measured up data, yours own results etc.– incl. theirs objectivity (principles of energy and mass conservation too).

- In your proposals calculate with an uncertainty and inaccuracy of data.

- In your proposals calculate with an irresponsible human factor (copperhead) and/or unthought troubles or process problems (in a solved line or in an others  one) ® fool-proof machinery.

- Take into account difference between theory and practice (k values for evaporators, power requirement for stirrers).

- Approach to solution has to be global (acquisition costs, running costs, work difficulty, maintaining time consumption, service and spare parts availability, benefits and savings, ecology, future aspects (output, legislative etc.) ...).

- A line reconstruction design in several steps (1st step = a worse line problems correction; low money, high effect; 2nd step = optimisation of a line; 3rd step = a correction of failings or problems of 1st and 2nd steps; final optimisation).

Philosophy of production lines design               sheets 10-12

# Basic terms

- Production line = serviceable and efficient group of production equipment (apparatuses or machines) in which is a technological process realised. The system is characterised by a flow of mass, energy and information.

- Technological process = organised sequence of physical, chemical, biochemical changes of substances of purpose to make a product (semi-product etc.) of required quality. T.p. is divided to operations (steps).

- Technology of production = process of technology (technology) describing a process how is a final product made from a raw materials.

- Operation = the smallest part of a process from a point of view of its time and space layout. Operation is usually a complex of several physical or chemical etc. processes (distillation, evaporation, drying = heat and mass transfer and flow; packaging = mechanical op.).

- Machinery  = machines and apparatuses in which is realised a technological process.

- - Working machines – mostly mechanical action to material (product)

- - Apparatuses – a special space with defined physical, chemical etc. process conditions

- Know-how = a complex of all knowledge of a technology including its acquirement (job training).

- Production line output = is given in pieces/s, pieces/h – wrapping machines; kg/s; t/d – sugar factories; l/d – dairy industry; hl/year – brewery ....

- VL = production line = a system compound of sub systems and elements

- System = organised group of elements that are combined each other and serve for given duties. It is a set of objects and functions and has these basic characteristics:

- - System content = material (machines, apparatuses) necessary for its function (incl.

- staff and treating material)

- - System structure = space and organising arrangement (vertical, horizontal, seriál,

- parallel ..)

- - Communication = equipment for transport of material, energy and information

- among single elements of a system (line) – piping, conveyers, staff, cabling ...

- - Control = insure a proper process of a technology from a point of view of quality,

- output, time path and sequence with other elements or processes in a line.

- Control is made by a staff or automatically on a various level and combination

## Reserves

- Technological reserves – are necessary for a technological process (cakes of cheese or pieces

- of meat in ripening cellars or boxes, beer or vine in fermentation tanks)

# Continuous reserves – are necessary from organisation reasons for a maximal utilisation of all

- line elements

- - Balancing reserves – equalises output and input variations among machines and

- apparatuses in line, between continuously and periodically working apparatuses etc.

- - Transport reserves – material in piping, on conveyers ....

- Safety reserves – are necessary for a good line working in a case of  some troubles of an

- element (time for a reparation etc.)

- Example: Technological processes and machinery in meat industry   sheet 13

- Process					Realisation

- Technology of meat processing		Butchery factory

- - cutting					- cutting room

- - fresh meat					- packing

- - smoked meat products			- smoked meat manufactory

- - boiled meat products			- boiled meat manufactory

- Technology of smoked meat production	Line of smoked meat

- - salami					- salami line

- - bratwurst					- bratwurst line

- - frankfurters					- frankfurters line

# Technology of frankfurters production	Line of frankfurters

- - weighing ® disintegration ®		scale ® cutter ® mixer ® filling machine

- mixing ® filling in covering ®		® smoking-chamber ® boiling -chamber

- smoking ® boiling ® cooling		® cooling-chamber

- Physical process of boiling:			Process and strength calculations, setting of

- - unsteady heat and mass transfer		process parameters

- Biological process of boiling:

- - micro-organisms growth and

- activity stopping

- Production lines subdivision     sheet 14

- According material movement

- - periodically working - < outputs, special batch processes

- - continuously working - > utilisation of capacity, needs of > reliability + control

- According product character

- - serial – piece-goods,

- - mass – liquid or loose materials (grain, flour, ...)

- - infinitive – foil on a roll, extruded material ....

- According machinery

- - set of machines – mechanical operations (packing, manipulation ....mixing)

- - set of apparatuses – mostly chemical, physical and biochemical processes

- (heating, cooling, boiling, biotechnology, .... mixing)

- - set of staff and tools – craft work with or without mechanical instruments

- - combination

- According way of service and control

- - manual

- - mechanised

- - automated – partially, fully, computerised

- Automation reduces  physical effort but increases psychical stress; computerisation highlights staff only for a critical situation.

- Principles of production lines design        sheet 15

# Product production in required quality and quantity with minimal costs

- Mass wastes utilisation as a raw material in the same or other line

- Waste energy utilisation in the same or other line

- Minimal pollution to living environment

### Particularities of production lines in food industry

- Food chain = farmer ® food industry ® storage ® distribution ® consumer

- (® = transport + event. cooling), every change of one link can affect the others

- Quantity of various products and technologies

- Biological character of raw materials, semi-products and products ® limited durability,

- instability, various properties that change with time or locality, demands for

- hygiene and sanitation

- Individual consumer behaviour – tastes vary, products are in or out, effect of financial

- situations and health conditions of people ..... ® it is difficult to forecast it

- Production lines building = drawing up of a complete project and line building incl.

- auxiliary plants. A project sets main mass, energy and information parameters of

- a line,	space layout, specifying of  equipment, transport ways, control etc. and

- consequently its economy. A complex approach is important.

- Production line design steps – submitter, user   	sheet 16

- branch prognosis 			marketing study (for what type of product will be

- long-range goals			interest, in what price and quantity ....)

- product price relations

- new product development in a laboratory	- technologist

- checking on of the new product quality		- technologist

- technology proposal				- technologist + engineer (reality)

- checking on of the technology on a pilot plant – technologist + engineer (plant design,

participation on tests)

- marketing study – consumers interest correction and specification (quality, price and quantity of a

- specific product)

- investment intention = data for a designers (technologist, engineer, experts for automatic system

of control, energy and energy saving)

It contains:  output, line parameters, product quality, max. price of line and product, available raw

materials and energy, manpower, location and space, know-how, terms, service life,

ecological and legislative restriction, geologic conditions ......

Proposed technology process has to be optimal from a point of view of inlets and outlets and its control has to be easy ® a continual process demands a reliable measuring of parameters of process, inlets and outlets etc. and reliable control components.

Production line design steps – designer	sheet 17

(technologist, engineer, experts for automatic system of control, energy and energy saving, building engineer)

Work flow:

- background research

- results application to the specific conditions of solved problem

- comparison with the submission – critical submission checking

- critical checking of conventional processes and technology

- possibilities of improvement (energy, raw material, quality, savings, environment, control, work safety ....)

- selection of an optimal version – relationship to other lines, flow sheet

- reservation of machinery – purchase, development and design, modification ....

- economical evaluation – product and line price, investment return .....

- processing of design documentation on various levels

Manufacturer:

Prototype design, production, trial operation, tuning, a documentation modification for a serial production.

Production lines supply is preferable to supply single machines or apparatuses ® a know-how is sold x guarantee for all line

Balance sheet of a production line (technology process)    sheet 19

# Environment

# Mass		Energy		Information

Dust, air, water, microorg.    Heat, light, vibration     Troubles in net, computer, electrostatic discharge ...

Inlets:				   Production line			Outlets:

Accumulation of:

# Mass				   Mass					Mass

Raw materials, auxiliary materials 	     Mass included in line			Products, auxiliary materials, wastes

(water, steam, cleaning agents

filtration materials, balast ...)

Energy				    Energy				Energy

Electric en., heat in steam, vapour,	      Change of enthalpy, kinetic or 		Fixed in product, aux. materials,

water, cold in water etc., 		      potencial energy of material			wastes

compressed air ....			      or line

Information			    Information				Information

Method of operating, controlled		       Inserted program, machines			Information of processes, quantity

parameters etc. – fill in a staff		       setting etc.				and quality of product, wastes etc

or a given program								control centre

Environment

# Mass		Energy			Information

Exhalations, dust, water 	Heat or cold dissipation 	Staff nonfeasance, machinery breakdown ...

steam, material escape ...	vibration, noise ...		® after-effects to all process, factory, accident ...

From most of line balances sheet follows that in products in principle is not an energy ® practically all inlets energy is dissipated to environment directly in heat losses or indirectly in wastes (cooling water, cooling towers, stack gases ....).

Levels of production lines control	sheet 20

- manual control

- 1 controller control 1 parameter to a set value

- 1 controller control 1 parameter depending on other parameter (steam flow x temperature in

- an apparatus, inlet milk flow x outlet milk concentration ...)

- mutual combination or possibly 1 controller monitors and controls more parameters

- computer optimises work of all line (monitors and evaluates all important parameters of line, gives

- off signals for actions for action parts (valves, butterfly valves, electric motors ....)

- It is necessary to count with a unreliable human factor and failure rate of machinery

- minimise service, maintaining and adjusting requirements (wrong service, inadequate

- maintaining, bad adjusting ® breakdowns)

- plan of maintaining – utilisation of theory of reliability

- diagnose a situation before a breakdown (temperatures, pressures, vibrations, leakage ....) ® well-

timed reparation in a proper time save big losses

- Complex line (technological

- process) design ® balance, process linking, machinery and

- its parameters design, local optimisation

# Optimal synthesis of line designs – effect of externalities (ecology, restrictions ...),

# optimisation  of the whole line

- Line design modifications – suggestions for the whole line optimisation and external

- conditions fulfilment

- Final synthesis of modified line (technological process) – definitive optimal design

of line fulfilling all conditions

Course of energy consumption in a dairy and a sugar factory  sheets 29,30

Branches of food industry   sheet 2

Meat industry

# Dairy industry

Mill and bakery industry

Sugar industry

Malt and brewing industry

Vine and spirits production

Fat industry

Poultry industry

Canning factory and ethanol production

Starch industry

Cooling and freezing plants

Chocolate etc. production

Tobacco industry

In CR food industry produced circa 14 – 15 % of all industrial production.

Food chain            sheet 3

agriculture ® transport ® food industry ® transport and distribution ® consumer

cooling	cooling	   cooling			 cooling

Family budget and foods

USA, Canada c. 15 %, EU c.15 – 25 %, CR 50 ® 40 ® 30 %, developing country till 95 %

### Food losses x wastes

developing countries till 80 % (hot weather, lack of technology), developed countries c. 20 %

Losses in the food chain: Agriculture c.16 %, food industry c.14 %, distribution c. 4 %, consumers c. 66 %

- wasteless technologies ® wastes are raw materials in other plant or other part of a line

- long-life products, good package

- manipulation, storage, transport etc. with regard to food material

- Optimisation of food chain = optimisation of 	- product

- - process

- - raw material properties

- Sugar: yield  in t/ha x sugar content in beets + composition (non-sugars)

- Corn: starch x gluten content

- Potatoes: for starch production x for boiling or potato salad x chips production

# Durability of foods, post-harvest life

- good durable – sugar, corn, flower, beans (soya, pea, bean ...)

- medium durable – potatoes, root crop (carrot, parsley ....), apples, hard cheeses, eggs ...

- with artificially prolonged durability – dried products, canned food

- perishable – milk, meat, fishes, vegetable, soft fruits ....

# For durability has effect: - composition and water content

- - way of storage (° C, % r.h., O2, light, contamination ....)

- -  manipulation

- - previous processing

- Food composition                 sheet 4

- C, O2, H2, N2  		- proteins		building material

- - carbohydrates	energy			   macro-nutrients

- - fats			energy

- - water

- - minerals (ash) 	Ca, P, Fe, S, Na, K, Zn ...      micro-nutrients

- - vitamins 		biochemical reactions

- water content ® food durability, weight

- water activity   aW = pPfood / pPwater  [0 – 1]

- bacteria	size c. 0,3 – 10 mm;	needs  aW > 0,90      are accustomed to an

- yeast		size c. X0 mm;		needs  aW > 0,80      osmotic pressure 550 kPa

- fungi		size c. X00mm;	needs  aW > 0,75      (0,9 % solution of NaCl)

- Substitution of expensive animal proteins with cheaper vegetable ones (soya, egg white)

- Producer tests composition of raw material and products ® quality, price

- Hydrocarbons = starches, sugars, fibres (cellulose, lignin)

- Causes of food losses           sheet 5

- 1. Effect of micro-organisms

- Bacteria – unicellular micro-organism (X mm; aW  > 0,90), vegetative and non-vegetative

- form (spores); < 100 °C (< 140 °C)

- Yeast  - kind of fungus (X mm; aW  > 0,80), £ 60 °C; need sugars, N2, a little O2; quick

- propagation, good adaptation, sensitivity to an osmotic pressure (< 550 kPa)

- Fungi – mycelium and sporocarp (X00m; aW  > 0,75), £ 100 °C; pH = 3 – 6; needs

- moisture, air, a little nutrients  x dangerous mycotoxins

- Mould infection – food fibrillation with a fungi mycelium ® toxins production

- x sterilisation, << moisture, access of air prevention

- Fermentative processes – bacteria, yeast

- - fermentation = decomposition of non-nitrogen matter (starch, sugar ..) when CO2

- is produced (production of beer, vine, vinegar, proofing dough ...); taste and

- smell changes, but acidity not

- - souring = decomposition of organic matter, acidity goes up, gases are not in practice

- produced; milk souring = sour milk products, sauerkraut, sour cucumbers ...

- x sterilisation, refrigerators, freezers, access of air prevention, concentration increase

- Decay – nitrogenous matter decomposition = proteins

- - bad smell, NH3, CO2, H2S are produced

- - 1st step – alkaloids are produced but smell and taste are good ® very dangerous

- - 2nd step – total decay – disgusting but relatively harmless products

- 2. Effect of enzymes             sheet 6

- Enzymes are catalysts of chemical reactions (favourable ® biotechnology) that decays

- food; a very small amount is sufficient

- Enzymes are: 	- hydrolytic – starch hydrolyse to simple sugars

- - oxidising

- - reducing

- - fermentative

- Ferments activity depends on a temperature (graph – maximum is between 20 – 60 °C)

- 3. Effect of chemical reactions

- - reactions are inhibited by enzymes inactivation – oxidation, hydrolysis, reduction,

- other reactions

- 4. Effect of radiation

- - light, UV radiation, electromagnetic radiation,

- 5. Mechanical effects

- - peel, shell, skin = natural cover of food

- - inconsiderate manipulation ® surface layers damage, surface layers structure

- disturbing ® decay processes  x do not wash eggs before a storage

- 6. Uncontrollable decrease of water content

- - desiccation of fruits and vegetables x controlled dehydration (drying)

- ® controlled atmosphere during fruits or vegetable storage (temperature, humidity,

- content of O2, CO2, ethylene ....)

- Means for food durability prolongation               sheet 7

- Cleanness	- physical – mechanical impurities

- - chemical – washing water, detergents, disinfectants ...

- - bacteriological – micro-organisms

- Chemical preservation – adding of bactericidal medium natural (sugar, salt, CO2,

- ethanol, ethyl ...) or chemical man-made (formic acid, benzoic acid ..)

- Preservation with controlled dehydration – water content decrease ® micro-organisms

- activity stopping

- - filtration, membrane processes, evaporation, drying, freeze-drying (sublima-

- tion)

- - salt or sugar adding – osmotic pressure changes to a value unfavourable

- for micro-organisms

- Preservation by physical methods

- - high temperature – pasteurisation

- - sterilisation

- - low temperature – cooling (+ 5 - - 0,5 °C)

- - freezing (- 10 - - 30 °C)

- Goals of food industry          sheet 9

- To produce an eatable food from mostly uneatable raw material

- (corn, sugar beet, oily seeds etc.)

- To produce food with better quality, taste, digestible, longer durability etc.

- To conserve agriculture surpluses from harvest for year-long consummation

- 3 parts of food industry

- Primary		corn ® flour	= agricultural product ® semi-factured product (maybe

- eatable or uneatable)

- Secondary		flour ® bread	 = semi-factured product ® product

- Tertiary		milk ® dried milk = agricultural product or semi-factured product ®

- meat ® canned 	product with special properties (longer

- durability,  instant, ….)

### Foods

- healthy, ecological, natural = have to fulfil criteria of correct diet

- correct – vegetable, fruits, milk, cereals …. are theoretically correct but from some

- locality they may be not healthy (contain Pb, Cd, Hg, PCB …)

- with a special purpose, goal-directed = their content and properties are modified for

- a special people group (children, sportsmen, convalescence, special diets …)

- Goal: Healthy foods have be delicious and good-looking too.

- Productions lines of a dairy industry                                sheet 31

- Milk products:

- market milk (standardised milk) – fat content 2,0 % (semi-fat), 3,6 % (full cream,

- unskimmed)

- skimmed milk – fat content » 0 % (fodder for hogs or calves, diet …)

- concentrated milk – sweetened or unsweetened (”condensed milk”)

- dried milk – for drinks preparation, bakery industry, fodder ….

- cream with various fatness

- cultured-milk products – yoghurts, kefir, cream, buttermilk …

- curds, cheeses

- butter

- ice-creams

- whey (concentrated or dried) – for animal fodder, food fortification ….

- lactose (milk sugar - dried) – food fortification, diets ….

- casein (milk protein - dried) - food fortification, diets ….

- additives to bakery products, smoked-meat products, shortenings ….

### Dairy location

- Near a source of raw material (milk production, grasslands, cow-houses …) or near a products consumption (big cities).

- Dairy types:

- Market dairy (chief products = market milk, cultured-milk, event. ice-creams…)

- Drying plant (chief products = dried milk, whey etc.)

- Cheese making (chief products = curds and cheeses)

- Butter factory (chief products = butter)

- Universal dairy (produces miscellaneous products)

- All the types of dairies product market milk for people round the dairy too.

- Milk content:

- water – 85 – 89 %

- dry matter 		- lactose  5,6 – 9,6 %

- - fat  2,7 – 5,0 %

- - casein  2,0 – 3,2 %

- - minerals  0,6 – 0,9 %

- - albumin, globulin  0,4 – 0,8 %

- - other organic matter  0,1 – 0,3 %

- Fresh milk quality                               sheet 35

# Fresh milk contains c. 200 – 2 000 000 micro-organisms / ml

- Bactericidal phase – c. 1 – 3 h after milking (milk drawing)

- in the time micro-organisms do not breed ® in the time it is necessary to

- filter and cool down milk c. to 5 °C ® cooled milk collections near farmers

- Milk acidity   	- 5,8 – 7,0 °SH - good quality

- - > 8 °SH – on heat transfer area begins to agglomerate (forms deposits)

- - > 10 °SH – pasteurization is not possible

- - > 12 °SH – milk agglomerates during boiling in an open cooker

- - > 20 °SH – cool milk agglomerates in an open dish

- Milk fatness - ® butter ® price

- Micro-organisms content - ® milk quality after pasteurization

- Process of buttermaking                               sheet 38

- Fat in milk is in a form of small droplets. The fat droplets can contact together (agglomerate) only on places where are juts without phosphatide layer and on interface fat - air. The process of the fat droplets agglomeration is better when is higher fat concentration (fat droplets amount, droplets are more close), when fat droplets stroke each other and when is an optimal temperature. The agglomerates form butter grains.

- From whence it follows a churns and  buttermakers construction

- hand churns

- drum churns (barrel, cylinder, double-cone) with movable and immovable built-in baffles

- continuous buttermaker

- Churning time in a drum churn is c. 35 – 45 min and revolutions 20 – 45 1/min.

- Churning temperature:

- sweet cream 9 – 14 °C;  in winter >, in summer < (effect of milk composition owing to

- fodder kind; ripening time c. 2 – 4 h,

- soured cream  9,5 – 15 °C; ripening time till 24 h, but better quality

### Effect of a churning time

- too short time ® butter grains are < , in butter is > water, in buttermilk is fat

- too long time ® buttermilk and air are churned in butter ® butter is viscous (pasty)

- with low durability

- The butter after the churning is raw = is grainy, incoherent ® it has to wash and knead ® excessive free water is separated and butter grains coalesce (join). The same is a principle of a continuous buttermaking.

- Barley steeping                    sheet 44

- amount of steeping water » 40 % of barley mass

- steeping time 3 – 4 days

- for 1 t of barley it is necessary c. 2,2 m3 of steep tank volume

- barley grains have to respire ® aerating, water drainage, barley passing from one tank to

- an other one – 1/3 starting steeping, 1/3 aerating, 1/3 final steeping

- during steeping enzymes activate, dyes (pigments), tanstuffs leached out (partially sugars

- and minerals too) – c. 1 %

- steeped grain has not to sting (prick), has to be able to hook over a nail, inside is white

- and on a board writes

## Barley sprouting

- Goal: Formation of enzymes necessary for a starch saccharification

- Sprouting ends when grains are cracked (with small root and leaf). Cellulose from cell-walls is hydrolysed.

- Steps of barley sprouting – c. 7 days:

- wet barley – from steeping

- wet pile

- dry pile – after c. 48 hours

- dissilient (dehiscent) grains – a white point on a one grain end (= << root)

- young grain – small sprout (= root)

- green brewer’s malt – a root is long like a grain, a leaf germ like 2/3 – ¾ of a grain

# Two-floor malt kiln     sheet 46

- filing tubes

- belt conveyor for green malt

- regulating flap valve

- mobile malt tedder

- upper folding tray (floor)

- lower folding tray

- reservoirs for malt, conveyors

- fan

- air heater

### Malt kilning = malt drying

Enzymatic processes needed for taste, good smell and beer quality are finished and in a proper time are stopped. For dark beers malt is dried with > °C (for Bavarian beer is air inlet temperature c. 100 – 105 °C ® malt partially caramelize).

Malt quality:

- Well dried malt has to float on water level, bad dried settles down

- Moisture is c. 4 %

- Grains have to be brittle and white, full

- Bad dried grain after heating form gelatinous structure and after drying harden ® does

- not soak

- Fermentation cellar     sheet 51

- Fermentation stages:	- dusting – like white powder on wort level

- - white rings on wort level

- - high brown rings (foam)

- - squirming rings

- - brown cover (brown foam on level)

- Fermentation time is c. so long in days as is beer percentage (sooner, now is shorter – Staropramen 10 % c. 7 days, 12 % c. 10 days)

### Cylindric-conical tank

- Substitutes fermentation cellar + storage cellar

- Stages:		- wort filling event. in pre-fermenting CC tank

- - inoculation; at 7 – 14 °C = brewer’s yeast adding

- - quick cooling to c. 0,5 °C

- - settled yeast withdraval

- - final fermentation

- Total fermentation time is c. 2 – 3 weeks

- > performance, < built-up place, no cellars, easy CO2 recovery

- Yeast dosing in wort                             sheet 52

# The system is used in Braník brewery – yeast is automatically dosed in wort flowing to CC tanks. The dosage is controlled by a mixture colour.

- In England is used so call ”top fermentation” at 20 – 25 °C ® quick fermentation but in beer (wort) are foreign microbiological cultures ® special taste for us

- In Europe are cultivated yeast cultures for low temperature ® no foreign cultures ® no the tastes

- It is possible to use yeast £ 4 times ® else is higher number of dead cells ® autolysis ® < quality

- Rest yeast is used as a fodder or for a special production (dried – tablets Pangamin)

### Storage cellar

The secondary fermentation takes place in storage tanks – 2 steps:

- Secondary fermentation at atmospheric pressure (a tank valve is open) ® bad smell from beer is removed

- Secondary fermentation at overpressure (a tank valve is closed) ® produced CO2 is absorbed in beer – CO2 content increases to c. 0,3 – 0,4 % (mass) ® beer gains its frothing quality

Beer 10 % is in storage cellar tanks c. 3 – 5 weeks		x   CC tanks 2 – 3 weeks for both

Lager beer 12 % c. 2 – 3 months 				     fermentations

< °C and > t ( but < 3 months) ® > beer quality; longer time is not  suitable  (yeast autolysis)

Beer is tapped from several tanks together = beer blending (quality equalisation) - a cellarman on the bases of laboratory tests and degustations sets what tanks will be mixed

Terms used in sugar industry technology              sheet 54

# Sugar = saccharose, sucrose

Raw sugar – purity 97 – 98 %  (3 – 2 % of non sugars)

Affinated sugar, affinade – purity 98,5 – 99,0 %

Refined sugar – purity 99,8 – 99,9 % (0,2 – 0,1 % of non sugars)

Digestion Dg (%) – mass ratio of sucrose in sugar beet

Saccharisation S (%) – mass ratio of dry matter in solution (juice)

Polarisation P (%) – mass ratio of sucrose in solution (juice)

Quotient of purity, purity Q (%) – mass ratio of sugar in dry matter

Non sugars NC (%) – mass ratio of non sugars in dry matter

P = Q * S / 100		S = P + NC

In sugar technology are in calculations used co called ”quantities per 1 ton of processed beet” (% beet = % ř.). The reason is a possibility of easy comparison of sugar factories with various capacities.

For example: 	Steam consumption is 30 % ř.

- For a sugar factory with a capacity of 4000 t beet / d is actual steam consumption

- (30 / 100) * (4000 / 24) = 50,0 t / h

- For a sugar factory with a capacity of 1500 t beet / d is actual steam consumption

(30 / 100) * (1500 / 24) = 18,75 t / h

## Sugar factory types

- raw sugar factory	= sugar beet ® raw sugar or affinade

- white sugar (refined sugar) factory = sugar beet ® raw sugar ® refined sugar

- white sugar factory with raw sugar purchase = sugar beet ® raw sugar own + raw sugar purchased ® refined sugar

- dry refinery = raw sugar purchased ® refined sugar

Thick juice or liquor boiling to a massecuite (batch vacuum pans)        sheet 63

- Vacuum pan evacuation

- Drawing in a boil groundwork c. 300 mm above upper tube-plate (thick juice, liquor, syrup)

- Boil groundwork concentration to a needed supersaturation (Kp » 1,2) with further drawing in of thick juice and liquor (for A product) or thick juice and yellow syrup (for B product) or black syrup (for end product)

- Crystals composition

- - seeding with a micro-seed (for 1 pan c. 300 ml of powdered sugar grain size c. 1 mm

- in ethanol)

- - adding of an artificial massecuite made from end product sugar = end product sugar

- mixed with thick juice (only for raw sugar or B sugar production)

- - hand made crystals creation = a quick pressure change results in a creation of big

- quantity small crystal nucleus (seeds)

- - batching = in the supersaturated sugar solution is added c. X00 kg of sugar (siftings

- from white sugar sorting sieves (for refined big crystals production)

- Seed (small crystals) boiling up – each small crystal is a basis of a new growing crystal (crystals quantity in seeding and after boiling up have to be the same). When is Kp too >  ”false grain (crystals)” form (boil powdering) ® the false grain has to be dissolved in water or light juice (® energy loses as the water has to be once more evaporated); during boiling up are gradually drawn in thick juice or liquor and so is kept needed supersaturation Kp; boiling up ends when is level c. 1,5 m above upper tube-plate

- Drying of boil – a massecuite (magma) = mixture of crystals and supersaturated solution is concentrated to c. 91 – 94 %; it has contain crystals of needed size and has not to be sticky

- Boil discharging – the massecuite is discharged in crystallizer where crystals for some time during cooling growth (part of sugar from solution passes to crystals ® higher sugar yield)

- Vacuum pan and piping steaming – hot steam removes rest of sugar solutions and crystals

Boiling time: 	- A product (raffinade)  c. 2,0 – 2,5 h

- B product or raw sugar c. 4 h

- C product (end product sugar) c. 16 – 20 h

® fluctuation of steam (vapour) consumption in boiling plant (boiling house) ® it is

necessary to co-ordinate work of all vacuum pans in boiling plant to be the total steam (vapour) consumption steady  (x continuous vacuum pans)

Boiling process control:

- manually – (massecuite testing on glass (crystals size and quattity), fingers (stickiness) , sight glasses in vacuum pans)

- automatically – refined sugar according conductivity, end product according viscosity

- Crystallisers – time of massecuite ageing is X h; horizontal half-cylinders with band mixer

- (batch) or vertical cylinders (continuous); natural cooling from level or cooling

- tubes, discs etc.

- Fully automated centrifuges ARO – filling of a basket with massecuite at c. 100 rpm, run up

- to centrifuging speed, centrifuging of syrups and washing-up (with condensate, syrup

- or steam) at c. 1200 rpm, effluent separation or not, slow-down to ploughing speed,

- ploughing of sugar at c. 500 rpm; S c. 3 min.

- Lines in mill industry  - flour production           sheet 71

# Grain

- floury kernel (endosperm) – starch, proteins; is covered with an endospermal layer compounds of proteins and fats

- germ (sprout) – fats, sugars, proteins, active enzymes, vitamins B1, E

- covering layers (glume) – cellulose, hemicellulose; protect germ and endosperm from dehydration and damage

- grain contains – c. 12 – 18 % of water; at the moisture is in inaction

- grain quality – is evaluated according to content of carbohydrates (starch, sugars, dextrin, cellulose, hemicellulose, pulp) and gluten (proteins insoluble in cold water)

- gluten – viscous and elastic matter; after wetting swells and forms an elastic lattice structure (net structure) ® + CO2 from dough proofing ® elastic and porous dough structure; in wheat grain is c. 14 % of dry gluten

## Grain milling

= separation of floury kernel (endosperm) from covering layers and germ

Work with a dusty material ® all connections etc. of machines, hoppers, pneumatic handling etc. have to be closed (cloth sleeves) and grounded; electric-motors have to be flameproof

Roller mill with two-pairs of rollers                             sheet 75

- grain (semolina) inlet

- feeding rollers

- grinding rollers 	– grooved for scraping or coarse milling

- - smooth for fine milling

- - gap and pressure are controlled

- scrapping brushes

- flour (semolina, pimple) outlet

High-speed roller is firm, low-speed roller is movable (gap and pressure control)

Rollers diameter is c. 250 – 300 mm, L » 1000 mm, hardness is HB = 450 – 520

Speed ratio: high-speed / low-speed = (3 – 7) / 1

Ex.: for wheat groats – circumferential speed is c. 6 m/s, for capacity 2000 – 7000 kg/h is electric-motor input c. 5,5 – 23 kW (for rye is the capacity half at the same el. input)

### Plansifter

Plansifter is used for classification of grist (flour) by means of plane sieves

Plansifter is hanged on wooden (laminated) bars and eccentric drive oscillated it to a wheal motion

The best flour is from the first milling (majority of endosperm), from next milling is in flour more particles of covering layers

Flour extraction (yield from grain): 	- c. 77 % flour + semolina + pimples

- c.  7  % shorts (flour for feeding)

- c. 15 % brans – feeding

- c.  1 %  sprouts (pharmacy), dust (feeding, compost)

Lines in bakery industry                           sheet 77

# Flour quality	- according Czech standards flour moisture has to be £ 15 %

- fresh milled flour has to age else is a dough greasy and sticky, creeps

and it does not heave (dough proofing)

- wheat flour ages 3 – 6 weeks, rye flour 1 – 2 weeks

- the higher temperature and moisture the shorter ageing time

Bread consumption in CR c. 36 kg/person/year, rolls and pastries c. 22 kg

### Basic bakery products

- Bread – rye, wheat, combination, whole-bread; bread forms – loaf, roll, formed …

- White products = rolls, French loafs (bagettes)  …; with fat or without fat

- Fancy cake – cakes, Christmas cakes, Easter cakes, pies, strudels, stuffed cakes … with or without filling

- Confectionery – tarts, desserts, rolls …

- Dry goods – crumbs, biscuits (store-bread) …

- Pastries – spaghetti, noodle, ….

- Line for bread production                   sheet 78

# Flour is stored in silos – there aged

- After ageing is transported in a daily silo

- Silos outlets are aerated and vibrated ® prevention of ”vaulting creation” during silos emptying

Leaven preparation (c. 20 h before 1st dough preparation)

- Base = crumble bread + rye flour + water; yeast from old bread cause dough aeration, bacteria typical taste; fermentation process c. 8 h at 24 – 25 °C

- I. stage of leaven = finished base + rye flour + water; ferments c. 5 h at 24 – 25 °C

- II. stage of leaven = finished I. stage of leaven + rye flour + water; ferments c. 4 h at 25 – 26 °C

- III. stage of leaven = finished II. stage of leaven + rye flour + water; ferments c. 2,5 h at 27 – 28 °C

- The leaven is used the whole week as so called reproduced leaven = 2/3 of leaven is put in a kneader + rye flour + water + salt solution + event. ingredients and all components are mixed and kneaded to dough (in dough is c. 56 % of reproduced leaven, c. 32 % of flour and c. 12 % of water); the spare 1/3 of the reproduced leaven is refilled with flour and water to a new reproduced leaven.

Processes proceed during bread baking        sheet 80

- starches and proteins swell, yeast escalate activity, maximum is at c. 35 °C

- at c. 50 °C yeast begin to wither away, starch begins to gelatinise and absorbs water, that loose proteins

- starch gelatinising is maximal at 80 °C, proteins coagulate ® a firm bread ”skeleton” (frame) is formed

- bread-crumb is during baking heated to c. 95 °C, crust to c. 170 – 190 °C

- at 100 °C starches begin to hydrolyse to dextrin, at 140 – 160 °C begin to caramelise sugars, fats and proteins change their nature too (denaturate) ® newly arose matters give to the crust a special taste, smell, colour and glance

### Processes proceed in baked bread

- after cooling bread contains c. 42 % of water

- the water during storage evaporates (1st day c. 3 %, next days c. 1 – 2 %)

- during storing and evaporating the amorphous gelatinised starch transforms to a starch with a crystallic structure = starch retrogradation

- water leaves bread-crumb (where was fixed in starch, proteins etc. structures) and goes to the bread surface = crust ® the crust loses its crackling; proteins transforms too ® bread quality deterioration

- Bakery ovens – dividing, types              sheet 82

- according baking area

- - peel ovens – stationary baking area, dough is put in and bread taken off by the help

- of a spatula – an old bread ovens in which were a fire-place

- - pull-out ovens – it is possible a baking area (baking tin) draw out from an oven

- - hanging plates ovens – on 2 chains are hanged ”benches” (plates) where is dough put

- and where is baked; inlet and outlet are usually at the same oven end (but in

- various heights)

- - belt ovens – the belt is from a wire-netting (screen); dough is put on the belt and

- during its moving through the oven is baked; bread outlet is at an opposite

- end than dough inlet

- according type of operation

- - periodical – peel and pull-out ovens

- - running (continuous) – hanging-plates and belt ovens

- according a way of heating

- - with direct heating

- - furnace ovens – old classic ovens; in a oven is fire and after some time when

- the oven is hot-heated ashes are raked out and in the oven is dough put

- - electric ovens – electric heating

- - with indirect heating

- - channel ovens – heating medium (liquid = dowtherm, delotherm  is heated

- outside the furnace) flows in tubes or channels and heats baking space

- - steam ovens – steam condensing in tubes heats baking space

- - cyclothermic ovens – flue gas from a burner or fire-place flows in heating

- channels and heats baking space; part of chilled gases recirculates and

- is mixed with a hot flue gas ® fan, fuel saving

- - convective ovens – baking space is heated by circulating hot air; heat transfer

- to dough is mostly by convection, partially by radiation from walls

- and conduction from baking plate ® > baking uniformity, baking time

- is shorter

## Small bakeries                              sheet 83

- electric pull-out ovens – dough can heave there and then is baked ® temperature control

- from 30 to 350 °C, possibility to wet working room, possibility of 2 working rooms

- (heaving, baking) separately controlled; specific output 10 – 20 kg/m2h

- batch type convective ovens – dough is put on plates like above + fan for hot air

- circulation

## Big bakerhouses

- continuous belt ovens – belt width till 3 m, length till 50 m

- - drawback – very long and low

- - majority of flue gas flows in channels under the belt = lower heat (the belt has to be

- heated and than hot air flows up)

- - minority of flue gas flows in channels above the belt = upper heat

- - heat of leaving flue gas is used for combustion air preheating, part recirculates,

- the rest is used for other heating purposes in a bakerhouse

- - front section of the oven (c. 25 – 30 %) has consumption c. 50 % of a total heat

- rear section has < heat consumption

- - specific output is till 20 kg/m2h (for bread or rolls)

- hanging plates oven – ”benches” (plates) width till 3 m

- - shorter and higher

- - more complicated flue gas and air flow

- - dough inlet and bread outlet is on the same oven side ® may be problems with

- manipulation and hygiene

- Extrusion                       sheet 84

- Physico-chemical process when materials (starches, proteins) change their structure

- (”frothing”)

- Material inlets to 1 or 2 worm(s) extruder; rotating worm(s) moves it to the other end

- where is smaller volume ® temperature and pressure increase (till c. 10 MPa and

- 200 °C); material is intensively mixed, starch becomes gelatious, proteins denaturate

- (semi liquid state of material, plastication)

- When the material outlets from a nozzle to an open air becomes superheated and steam

- leaves the material ® a typical porous structure rises

- Extruders are without or with heating (if a transformation of worms mechanical energy to

- heat is not sufficient)

- Extruded material is modified according needs – pressing rolls ® needed thickness,

- cutting, surface finishing (flavours, sauce …..), roasting, drying, cooling, packaging …

- Lines for starch production                             sheet 85

- Wheat starch was used as early as 3500 years B.C. in cosmetics, paper production and medicine

- Technology of a starch separation was described by Cato (c. 200 y. B.C.) – wheat milling, grist mixing with water, fermenting, washing of arose dough, drying ® product was a starch powder = amylum

- Analogous was a starch production from rice etc.

- First factory for starch production was in the U.S.A. in 1707 (from corn (maize))

- In 18th century was in Europe starch produced from potatoes

- From 19th century are produced so-called modified starches, starch derivates (+ chlorine, hydrogen peroxide, sodium peroxide, formaldehyde …)

## Starch and its properties

- Starch = a mixture of 2 polysaccharides (20 % of amylose and 80 % of amylopectin) organised in a complicated structure

- It is a white powder in cold water insoluble, enzymes do not act on it in the state

- When is water-starch dispersion heated above 60 °C (temperature of gelatination) starch particles swell up, crack and starch molecules dissipate in water; to the gelatinous starch enzymes can act

## Saccharides

- Monosaccharides

- - glucose (grape sugar) – C6H12O6  (aldehydic group CHO)

- - fructose (fruit sugar) – C6H12O6  (ketonic group CO)

- - xylose, mannose, galactose …

- Oligosaccharides (2 – 10 connected monosaccharides)

- - saccharose (sucrose, beet or cane sugar) = glucose + fructose – C12H22O11

- - lactose (milk sugar) = glucose + galactose

- - maltose (malt sugar)

- Polysaccharides (> 10 connected monosaccharides)

- - amylose – is compound of maltose units, in water is soluble

- - amylopectin – is compound of glucose units (C6H12O6)n , in water is insoluble

- - dextrins – on a glucose basis

- - inulin – on a fructose basis

- - cellulose – the biggest molecules – cell walls, supporting tissues

- Starch is made from potatoes, wheat, maize (corn), rice, manioca ….

- (in CR in 1994 was price of potatoes starch c. 15,- Kč/kg

- price of wheat starch c. 8,- Kč/kg)

## Potatoes starch production

- the best quality but the most expensive production

- potatoes with high starch content and resistant to infections was cultivated

- campaign production

- potatoes dry matter losses in waste water are till 25 % ® charge for environment

- world capacities are 1400 – 3600 t/d of potatoes, in CR only 100 – 600 (1000)

## Maize starch production

- special maize sorts were cultivated (100 % of amylopectin or 100 % of amylose etc.)

- technology is practically perfect

- the cheapest production, big capacities, year-long production

## Wheat starch production

- special wheat sorts were cultivated (starch content, gluten content, yield …)

- technology is similar like for maize

- starch quality is a bit worse

- Starch production and its processing is a quick developing branch of industry

- Potatoes starch production                          sheet 87

# Potatoes contains c. 76 % of water and c. 15 – 18 % of starch

- Starch granules are closed in potato cells. They are released by grating. Released starch granules are separated from pulp and proteins by washing and sedimentation. A starch refining (purification) is provided by a washing. Then is starch dried, milled and sifted. In the process are mostly used hydro-mechanical processes.

## Starch milk refining (purification) and processing

- Pulp separation

- Starch milk contains water, starch granules and a small quantity of very fine pulp. The pulp is separated by filtration or sedimentation processes ® vibrating purifying sieves, curved sieves or washing centrifugal machines (type Starcosa)

- Starch milk concentration

- - quiet milk sedimentation 			   vessels, mangers, chamber traps

- - flowing milk sedimentation 		    in old factories

- - starch sedimentation in hydrocyclones	   in new factories

- - starch sedimentation in centrifuges

- Starch washing

- - starch in a starch milk is not clear ® is washed with fresh water in mixed vessels

- (3 – 8 m3 + stirrer)

- - after c. 1 h of mixing is the stirrer lifted from the vessel and mixture sediments c.

- 10 h (several sedimentation layers forms; from bottom up = sand, bigger and

- finer starch granules, fine sludge + coagulated proteins)

- - the layers are separated by a special draining and washing is repeated

- - settled starch is mixed with water to starch milk with concentration c. 50 %

- Starch dewatering

- 1° mechanically = filter horizontal centrifuges, semiautomatic (shell from a perfo-

rated plate + sieve) or vacuum filters

- 2° drying to c. 20 % of moisture (starch temperature has to be < 40 °C to avoid

a starch gelatinising – heaving); dryers: drying hurdles in chamber dryers,

tunnel dryers, disc dryers, flow dryers, fluidised-bed dryers …

Flow sheet of potatoes starch production              sheet 88

potatoes

¯

water   ® 	storage

¯	potatoes

water   ®	potatoes cleaning, weighing	® 	catchers, washers like in sugar factory

¯	sand, stone, leaves – 10 – 30 %

¯	potatoes

water   ®	potatoes grating			grater, desintegrator

0,03 % of H2SO3  ®	    ¯

per grated potatoes	      ¯	pulp (grist)

water   ®	pulp washing		®  waste water		sieves, centrifugal machines

and separation	®

¯   	washed pulp	   ¯ starch milk

water   ®	pulp re-grating				grater, desintegrator

H2SO3  ®

¯	washed pulp

water   ®	pulp re-washing	®  waste water + pulp	sieves, centrifugal machines

and separation

¯   	starch milk 2,0 – 2,5 °Bé

water   ®	starch milk purification  ®  fine pulp	vibrating sieves

¯   	starch milk

starch milk concentration  ® water		hydrocyclones, centrifuges

¯   	starch milk c. 15 – 20 °Bé (50 % D.M.)

water   ®	starch milk washing  ®  fine pulp etc.	mixed vessels

¯   	starch milk

starch milk filtration   ® water		centrifuges, filters

¯   	wet starch

hot air  ®	starch milk drying   ® air + vapour			dryers

¯   	dry starch

starch treatment (milling, sifting, expedition)	mill, plansifter …

¯

dry starch

Pulp (třenka) = starch granules (c. 10 %), potatoes water, grated tissue, added water and proteins.

Washed pulp (zdrtky) = pulp without majority of starch granules.

Water consumption is c. 8 – 10 m3 / t potatoes

Flow sheet of potatoes starch production – an old system             sheet 89

- potatoes reception and storage		14. pre-refining vibrating sieve

- potatoes pump				15. starch milk and pulp separator

- potatoes washer				16. refining vibrating sieve

- bucket elevator				17. refining settlers (alternatively)

- scale					18. mixed tank with circulation

- bunker					19. mixed tanks – settlers

- grater (desintegrator)			20. tank for concentrated starch milk

- hole for pulp				21. vacuum filter

- pulp separator				22. filter centrifuge (alternatively)

- 1° of washing				23. starch disc dryer

- re-grater					24. plansifter

- 2° of washing				25. starch expedition

- pulp-press

Flow sheet of maize starch production  (simplified)            sheet 92

maize	100 kg

¯

storage + cleaning				silos etc.

¯

water from technology®	steeping	® steeping water		steeping tanks

H2SO3 	  				   ¯  concentration in film evaporator

¯		        + drying	c. 6 kg

raw starch milk	¬	grinding and					disintegrators, hydrocyclones

sprouts separation	® sprouts

¯  pressing, drying  ®  oil, feeding	c. 8 kg

¯

fine milling					mills

¯

raw starch milk  	¬	peels separation	® peels			sieves

¯  milling, pressing, mixing, drying – pulp  c. 10 kg

gluten separation	® gluten water		filters, centrifuges

¯  settling, evaporating, drying

¯				gluten = protein from maize	c. 6 kg

water  ®	starch washing  	®  water		mixed vessels, centrifuges …

¯		®   liquid starch

hot air  ®	starch separation  ® water

and drying	®   air + vapour

¯

dry starch	c. 70 kg

Wheat starch production is similar.

Water recirculation: cleaning, pasteurisation, SO2 adding ® < water consumption (warm close method in an acid media)

Waste water treatment                        sheet 93

Waste water quantity in old starch factories is c. £ 1000 % of treated potatoes; now economical technologies are used with multiple water using  x  problems as the water contains organic matter, salts etc., decays, froths …

Ways of treatment:

- fields irrigation – acceptable for small factories (soil salination ….)

- ponds with or without turbine aerators

- waste water treatment plans (2 stages – anaerobic + aerobic)

- membrane filtration ® water back to technology, sludge to w.w.t.p. (ČOV), concentrating, drying (® feeding)

- concentrating, evaporating, drying  ® feeding or fertiliser (? desalination)

- Starch treatment

- Hydrolysis in water = long starch molecules splitting to smaller ones

- starch = polysaccharides ® monosaccharides glucose + fructose = substitutional

- sweeteners

- degree of saccharification (Dextrose Equivalent) DE = 100 % - only pure glucose

- - acid hydrolysis - hydrochloric acid, sulphuric acid, oxalic acid … + temperature

- + pressure  ® molecules splitting, neutralisation, cleaning, concentration

- G-F syrups    DE » 30 – 55 (30 – 50 % of starch transforms to glucose and fr.)

- - combined hydrolysis – after acid h. follows saccharification catalysed by an enzyme

- amylogucosidase ® inactivation, filtration, cleaning, concentration;

- DE = 93 – 94

- - enzymatic hydrolysis – a starch saccharification and liquefying by the help of

- enzymes amylase and then amylogucosidase ® inactivation, filtration,

- cleaning, concentration; DE = 96 – 98

- C. 50 % of world’s starch production is used for sweeteners production

- Products from starch

- - G-F syrups = glucose-fructose syrups ® sweeteners with the same sweetening

- effect like sugar but with lower energetic content + better for digestion

- - crystallic glucose – for medicine

- - maltose syrup – candies production

- - technical dextrins – textile industry, additives to paints ….

- - malto-dextrins – clear solution, soluble at cold state, without any taste (for medicine,

- diets, lemonades, ice-creams, meat industry, processed cheeses production,

- durable pastry …..

- - starch and dextrine glues, starch latex, dextrine-latex glues – biodegradable

- material

- - modified starch – a special treatment ensures required and defined properties of

- a product

- - in food industry – extruded products, cereals, instant flours, additives to

- pastries, special nourishment ….

- - in civil engineering – bonding compounds in mortar made from fly-ash

- - geology – material that is plastic at very high shear stress

- Edible oils and spreads production                        sheet 95

- Fats (lipides) = compounds of fatty acids, fixed to a glycerine molecule

- Fats are insoluble in water

- According a way of fixing of adjoining carbon atoms fat acids are divided to:

- - saturated fat acid: bond C – C; animal fats, cholesterol, infarct, arteriosclerosis …

- - unsaturated fat acids: bond C = C; vegetable oils (fats)

- Molecule size and carbon fixing determine fat property

- - liquid at normal temperature – unsaturated, short molecular chain

- - solid at normal temperature – saturated, long molecular chain

- - for the same degree of saturation is a fat with longer molecule chain stiffer

- Sources of oils and fats                    sheet 96

- animal fats = lard …

- vegetable oils = vegetable seeds and fruits

- - rape seed – Europe

- - sunflower seed – Europe

- - soybeans

- - olives – south Europe

- - maize sprouts

- - palm nuts (coconuts, copra)

- - peanuts

- - cacao beans

- Vegetable oils are used in food industry, cosmetics, pharmacy, technology (lubricants, fuels, hydraulic oils – ecological).

- Cultivation provided plants with required raw material properties and consequently of oil too (health, nutritive, technological etc.aspects)

- Oily seed (fruit) contains from:

- oily part – cultivation is able to affect quantity and quality

- proteins part – cultivation is able to affect quantity and quality, forage-cake

- Oil is from seeds (fruits) gains by:

- pressing, squeezing – the oldest method – best quality (virgin oil), lower yield

- extraction – presently used – higher yield

- In CR is wide-spread oil production from rape-seed

- Varieties with lower content of erucic acid (causes worse oil properties) and linolenic acid (causes lower oil thermal stability) and higher content of linoleic acid (higher oil quality) were cultivated.

- Table oil or spreads production                      sheet 97

- raw oil production

- refining of raw oil – refined oil production – table oil

- stiffening of refined oil – spreads, margarine etc.production

- Table oil production from soy – approximate balance

- Inlets raw material: 	- 1000 t/d of soybeans (18 % of oil, 12 % of water)

- Products:		- 169 t/d of raw oil

- - 600 t/d of proteins concentrate (85 %)

- - 7 t/d of raw lecithin

- - 80 t/d peelings

- Storage:		- silos capacity c. 80000 – 90000 t (for 90 days production)

- Cleaning:		- screening, sifting, irons removing

- Beans preparation: 	- crushing, peels separation, tempering, milling

- Oil extraction:		- grist extraction, miscella (= solution of oil in solvent (hexane, petrol))

- miscella distillation (solvent recovery and oil separation), extracted

- grist stripping (solvent from grist recovery)

- Oil cleaning:		- phosphatides separation, dewatering, cooling  ® raw oil

- Raw oil refining:	- washing, bleaching, desodoration ® table oil

- Grist treatment:	- feeding, soy flour (from milled soybeans with peels (hulls), or from

- stripped grist), soy proteins for adding to meat  products (soy

- proteins are cheaper than animal ones)

- By the reason of better table oil quality rape seed + sunflower + soy oils are mixed together.

- Flow sheet of edible oil and spreads production                     sheet 98

- Pressing can separate till 80 % of oil from seeds (fruits). Press till 200 MPa, temperature till 160 °C.

- In soybean oil is c. 25 % of proteins and 3 % of phosphatides, in rape seed oil is c. 55 % of proteins and 1,2 % of phosphatides.

- Soap-stock is soluble in water but not in oil ® water + NaOH solution adding (raw oil neutralisation and soap-stock centrifuging)

- Bleaching clay absorbs colour impurities and then is separated (filtration)

- Deodorization = bad smell separation by processes of oil heating and stripping

- Tempering pan                                  sheet 99

- Function and purpose: - small oil droplets coagulation in bigger ones

- - proteins denaturation ® better oil and grist separation (yield)

- Semi-continuous desodoration                sheet 102

- Goal:		- Bad smell and taste compounds separation; their cause an oil inedibility;

- their amount is only 0,001 – 0,01 %

- Principle:	- distillation – volatile compounds evaporate and are drawn off with a stripping

- steam from a desodorator

- - temperature is c. 220 – 240 °C, pressure c. 460 Pa (= vacuum)

- - oil is on plates (floors) hold for some time (20 – 30 min. ® semi-continuous

- process) and then is passed down to a lower plate where is the process

- repeated

- - stripping steam is injected directly to oil and so helps to volatile compounds

- separation

- Stiffed fats (margarine) production                     sheet 103

- - It was developed in 1870 as a substitute (ersatz) of butter during Prussian – Austrian war

- - Advantages: less cholesterol, saturated fat acids, good spreading at low temperature,

- cheaper than butter

- Oil hydrogenation

- = an oil saturation with hydrogen = change of double bonds of carbon to single bonds

- oleic acid 					stearic acid

- H    H    H    H 				       H    H    H    H

- |      |      |      | 				        |      |      |      |

- R – C – C = C – C – R	   Þ	            R – C – C - C – C – R

- |                    |				        |      |      |      |

- H                  H				      H     H    H    H

- Hydrogenation steps: continuous, discontinuous (batch)

- reactors 5 – 20 m3, mixer, heating (cooling) coils

- oil filling, heating to c. 50 °C, hydrogen adding to eject air from oil

- catalyst adding (powdered Ni c. 0,1 – 1 % mass)

- heating to c. 150 – 160 °C, hydrogen supply in oil (c. 20 – 50 m3 / t of oil), pressure c. 200 – 300 kPa (maybe 1,5 – 3,0 MPa too ® quicker process), hydrogen has to be perfectly dispersed in oil

- exothermic reaction ® cooling – mixture heats to c. 180 °C for margarine and to c. 200 – 220 °C for fats for technical purposes

- non reacted hydrogen is cleaned and flows back to a gas-holder

- during hydrogenation are samples of fat taken for melting point testing ® determination of hydrogenation end

- fat cooling below 100 °C

- perfect filtering off Ni (Ni is a poison), centrifuging

- stiffening time is c. 3 – 6 h (for higher press lower)

Fat can be sprayed in hydrogen too (= contrary to above where was hydrogen injected in oil).

Composition modification

The stiffed fat is mixed with vegetable oils, dried milk, water, vitamins, emulsifiers, seasonings …..